Data Analytics Insights Report

Traffic Patterns on I-94 (Minneapolis–St Paul) — Smart City Mobility Analysis

1. Overview

This report summarises key insights derived from analysing approximately 48,000 hourly traffic records collected along the westbound I-94 corridor near Minneapolis–St Paul. The dataset includes traffic volume, weather conditions, temperature, time-of-day attributes, and federal holiday indicators. The goal is to support the Smart City Mobility Team in understanding congestion patterns, identifying behavioural trends, and informing data-driven mobility planning.

Three analytical themes guided the exploration:

- Temporal patterns (daily, hourly, seasonal)

- Environmental influences (weather, temperature)

- Congestion behaviour (traffic categories, peak periods)

These insights form the foundation for future predictive modelling and mobility optimisation.

2. Key Findings

2.1 Daily Traffic Trends (2015–2017)

Daily average traffic volumes show consistent commuter-driven patterns across all three years. Weekdays exhibit significantly higher traffic compared to weekends, with pronounced peaks during work periods. Year-on-year comparison indicates:

- 2016 and 2017 show slightly higher daily averages than 2015, suggesting gradual growth in corridor usage.

- Seasonal fluctuations are visible, with higher volumes during warmer months and reduced movement during winter periods.

Implication: Daily traffic behaviour is stable and predictable, making it suitable for forecasting and congestion-mitigation strategies such as dynamic routing or adaptive signal timing.

2.2 Hourly Traffic Trends (2017)

Hourly analysis reveals classic commuter peaks:

- Morning peak: 7 AM – 9 AM

- Evening peak: 4 PM – 6 PM

- Lowest volumes: 12 AM – 5 AM

Traffic volume is strongly tied to human activity cycles rather than environmental factors.

Implication: Peak-hour congestion management should remain a priority. Opportunities include ramp metering, real-time travel alerts, and demand-responsive transit scheduling.

2.3 Weather Condition vs Traffic Volume

Average traffic varies across weather conditions:

- Highest traffic: Clear / Cloudy / Mist

- Lowest traffic: Snow / Heavy Rain / Thunderstorm

- Difference: Typically several hundred vehicles between best and worst conditions.

However, weather effects are secondary compared to time-of-day patterns.

Implication: Weather-aware traffic models can improve prediction accuracy, but interventions should focus on commuter peaks rather than weather anomalies.

2.4 Temperature vs Traffic Volume (Scatter Plot)

The scatter plot shows:

- No strong correlation between temperature and traffic volume.

- Higher traffic clusters around 5°C to 20°C, reflecting typical daytime conditions rather than temperature influence.

- Outliers appear during extreme cold or warm periods, but these represent isolated events rather than systemic trends.

Implication: Temperature alone is not a reliable predictor of congestion. Models should prioritise temporal features (hour, day, month) over temperature.

2.5 Congestion Categories (Low, Medium, High)

Categorising traffic volume reveals:

- High congestion aligns almost perfectly with morning and evening peaks.

- Medium congestion dominates mid-day periods.

- Low congestion occurs overnight and early morning.

Weather and temperature have minimal impact on category shifts.

Implication: Congestion levels are behaviour-driven. Smart-city interventions should target peak periods with demand-management strategies.

3. Strategic Implications for Smart City Mobility

3.1 Predictive Traffic Management

The stability of daily and hourly patterns makes this corridor ideal for:

- Machine learning–based congestion prediction

- Automated incident detection

- Dynamic routing recommendations

Temporal features should be the core of predictive models.

3.2 Infrastructure & Policy Planning

Insights support:

- Prioritising road maintenance during low-traffic overnight windows

- Enhancing public transit frequency during peak hours

- Deploying variable message signs (VMS) for weather-related advisories

3.3 Commuter Experience Enhancement

Data suggests opportunities to:

- Provide personalised travel time predictions

- Offer alternative route suggestions during peak congestion

- Integrate weather-aware travel alerts (e.g., snow or heavy rain)

4. Conclusion

The analysis confirms that commuter behaviour is the dominant driver of traffic volume on I-94. Weather and temperature exert secondary influence, while congestion patterns remain highly predictable across years. These insights provide a strong foundation for developing intelligent mobility solutions, including predictive modelling, adaptive traffic control, and commuter-centric travel tools.

The Smart City Mobility Team can leverage these findings to design targeted interventions that reduce congestion, improve travel reliability, and enhance overall mobility efficiency across the region.